Configuración actual de CubeMX: Firmware y periféricos

Las siguientes configuraciones deben seguirse en orden.

No se debe modificar nada que no se diga en este archivo, por lo que si al realizar una configuración de las mencionadas aparecen más opciones, dejar como están por defecto las mismas.

RCC:

- LSE: Crystal/Ceramic Resonator.

SYS:

- Timebase Src: TIM6

ICACHE:

- Mode: 1-way (direct mapped cache).

ADC1:

- IN1  IN1 Single-ended.

- IN2  IN2 Single-ended.

- IN3  IN3 Single-ended.

- IN4  IN4 Single-ended.

- Temperature Sensor Channel  ON.

- Vrefint Channel  ON.

- Clock Prescaler: Asynchronous clock mode divided by 2.

- Resolution: ADC 12-bit resolution

- Scan Conversion Mode -> Disabled

- Continous conversión mode -> Disabled

- Discontinuous Conversion Mode -> Disabled

- Enable Regular Conversions: Enable.

- Rank 1  Channel: Channel Temperature Sensor.

Sampling Time: 391.5 Cycles.

Offset Number: No offset.

LPTIM1:

- Mode: Counts internal clock events.

- Clock  Clock Prescaler: Prescaler Div16.

- Counter  Period: 65635.

Repetition counter: 0.

- LPTIM1 global interrupt: Enabled.

LPTIM3:

- Mode: Counts internal clock events.

- Clock  Clock Prescaler: Prescaler Div1.

- Counter  Period: 65635.

Repetition counter: 0.

- LPTIM3 global interrupt: Disabled.

LPTIM4:

- Mode: Counts external Clock events with internal synchronization.

- Clock  ULP Clock Polarity: Rising Edge.

- Counter  Period: 65635.

Repetition counter: 0.

- LPTIM4 global interrupt: Disabled.

RTC: Descipcion

- Activate Clock Source: ON.

- Activate Calendar: ON.

- WakeUp: Internal WakeUp.

- Hour format: Hourformat 24.

- Data Format: Binary data format.

- Hours: Hora actual.

- Minutes: Minutos actuales.

- Seconds: Segundos actuales.

- Month: Mes actual.

- Date: Día actual (número).

- Year: Año actual.

- Wake Up Clock: RTCCLK/16.

- Wake Up Counter: 4096.

- NVIC Settings  RTC non-secure interrupt: Enabled.

SPI1: Se conecta con el IC driver del LCD PCF8553. El microcontrolador solamente transmite datos. Se busco tasa de transferencia alta, para no estar mucho tiempo fuera de estados de bajo consumo.

- Mode: Half-Duplex Master.

- Data Size: 8 bits.

- Prescaler (for Baud Rate): 16.

- Baud Rate: 1.5 MBits/s.

- CRC Calculation: Disabled.

- NSSP Mode: Disabled.

- NSS Signal Type: Software.

USART1: Se conecta al conector STLINK con propósitos de debuger.

- Mode: Asynchronous.

- Baud Rate: 115200 Bits/s.

- Word Length: 8Bits.

- Data Direction: Receive and Transmit.

USART2: sin implementar, pero se reserva los pines y se lo cablea a placa de usuario.

USART3: Se lo usa para conectar modulo IOT.

- Mode: Asynchronous.

- Baud Rate: 115200 Bits/s.

- Word Length: 8Bits.

PWR:

- Low Power  Dead Battery Signals disabled: ON.

Power saving mode: ON.

- System power supply  Power Regulator: SMPS.

Middleware and Software Packs  THREADX:

- Core: ON.

- TraceX Support: ON.

- Low Power support: ON.

- Low Power  Enable LowPower Support: Enabled.

TX_LOW_POWER_TIMER_SETUP: app_threadx_lowpower_timer_setup.

TX_LOW_POWER_TICKLESS: Disabled.

TX_LOW_POWER_USER_ENTER: app_threadx_lowpower_enter.

TX_LOW_POWER_USER_EXIT: app_threadx_lowpower_exit.

TX_LOW_POWER_USER_TIMER_ADJUST: app_threadx_lowpower_timer_adjust.

Clock Configuration:

- System Clock Mux  MSIS  SYSCLK: 24 MHz  AHB Prescaler: /1.

- ADC,DAC Clock Mux  SYSCLCK: 24 MHz.

- SPI1 Clock Mux  SYSCLCK: 24 MHz.

- USART1 Clock Mux  SYSCLK: 24MHz.

- RTC Clock Mux  LSE: 32.768 KHz.

- LPTIM3,4 Clock Mux  LSE: 0.032768 MHz.

- LPTIM1 Clock Mux  LSE: 0.032768 MHz.

Pinout view:

Pines del micro no conectados sin configuración.

Pines conectador a un port undefined configurarlos como GPIO_Analog y label genérico, es decir nombre del puerto.

- PA1: SPI1_SCK  PCF8553_SCL

- PA4: GPIO_OUTPUT  PCF8553_CE

- PA6: GPIO_OUTPUT  PCF8553_RESET

- PA7: SPI1_MOSI  PCF8553_SDIO

- PA9: USART1_TX

- PA10: USART1_RX

- PA13: DEBUG_JTMS-SWDIO

- PA14: DEBUG_JTCK-SWCLK

- PB3: DEBUG_JTDO-SWO

- PB5: GPIO_OUTPUT  LED_ERROR_HANDLER

- PB6: GPIO_OUTPUT  LED_MCU_RUN

- PB7: GPIO_OUTPUT  LED_3

- PC0: ADC1_IN1  RTD_IN

- PC1: ADC1_IN2  BATTERY_LEVEL

- PC2: ADC1_IN3

- PC3: ADC1_IN4

- PC14: RCC_OSC32_IN

- PC15: RCC_OSC32_OUT

- PD8: USART3_TX

- PD9: USART3_RX

- PD13: LPTIM4_IN1  PULSE_IN

- PD14: GPIO_EXTI14  PULSE_IT

- PE10: GPIO_EXTI10  KEY_DOWN

- PE11: GPIO_EXTI11  KEY_UP

- PE12: GPIO_EXTI12  KEY_ENTER

- PE13: GPIO_EXTI13  KEY_ESC

NVIC:

- EXTI Line10: Enabled  Preemption Priority: 5.

- EXTI Line11: Enabled  Preemption Priority: 5.

- EXTI Line12: Enabled  Preemption Priority: 5.

- EXTI Line13: Enabled  Preemption Priority: 5.

- EXTI Line14: Enabled  Preemption Priority: 5.

- LPTIM1 global interrupt: Enabled  Preemption Priority: 0.

- RTC non-secure interrupt: Enabled  Preemption Priority: 0.